The Saeima1 has adopted

and the President has proclaimed the following Law:

Construction Law

Section 1. Terms Used in this Law

The following terms are used in this Law:

1) construction permit – an administrative act with conditions for implementation of a construction intention on site – for designing and construction work – until accepting of a structure for service;

2) construction work – part of the construction process, work carried out at a construction site or in a structure, in order to create a structure, to place a pre-manufactured structure or part thereof, to rebuild, renew, restore, preserve, arrange or demolish a structure;

3) structure – a physical object which has resulted from human activities and is linked to a foundation (ground or bed);

4) renewal of a structure – construction work as a result of which the load-bearing elements or constructions of a structure have been changed or functional or technical improvements have been performed without changing the dimension of the structure or the load-bearing capacity of elements;

5) arranging of a structure – construction work which is performed for assembling, laying or placing of an engineering structure in the foundation or structure;

6) preservation of a structure – construction work which is performed for strengthening and protection of the structures of a building against unfavourable external exposure so that the structures of the building do not reach a condition dangerous to human life, health or the environment, as well as for disconnection of the engineering networks built;

7) demolition of a structure – construction work as a result of which a structure or part thereof ceases to exist;

8) placing of a structure – construction work, which is performed for assembling of a pre-manufactured structure from ready-made elements in the intended place, without building a foundation or basis in depth of more than 30 centimetres;

9) rebuilding of a structure – construction works as a result of which the dimension of a structure or part thereof are have been changed or the load-bearing elements or constructions have been strengthened, with or without changing the type of use;

10) restoration of a structure – construction work, which is performed for a scientifically justified renewal of a structure or part thereof, using materials, methods or technologies corresponding to the original;

11) construction product – any product or industrially produced construction that is intended to be incorporated into a structure;

12) construction – designing of all types of structures, and construction work;

13) building design – an aggregate of graphic and text documents necessary for the implementation of a construction intention.

Section 2. Purpose of this Law

The purpose of the Law is to create a living environment of good quality, determining efficient regulation of the construction process in order to ensure sustainable State economic and social development, preservation of cultural and historical and environmental values, as well as rational use of energy resources.

Section 3. Scope of Application of this Law

The Law shall be applied to construction of new structures, as well as to rebuilding, renewal, restoration, demolition, placing of existing structures, to changing of the type of use without rebuilding and to preservation.

Section 4. Construction Principles

Construction shall include designing and construction work, as a result of which existing structures are improved or demolished or new structures with a specific function are created. The following principles shall be conformed to in construction:

1) the principle of architectonic quality, according to which structures are designed balancing the functional, aesthetical, social, cultural and historical, technological and economic aspects of construction, as well as the interests of the commissioning party and the public, emphasising the individual identity of nature or urban landscape and organically integrating it into the cultural environment, thus enriching it and creating a living space of good quality;

2) the principle of engineering technical quality, according to which the engineering technical solution of the structure is safe for use, as well as economically and technologically efficient;

3) the principle of openness, according to which the construction process is open, the public is informed regarding the anticipated construction and the decisions taken in relation thereto;

4) the principle of sustainable construction, according to which a living environment of good quality for current and future generations is created during the construction process, increasing efficient use of renewable energy resources and promoting efficient use of other natural resources for such purpose;

5) the principle of environmental accessibility, according to which such environment is created during the construction process, in which any person may move with comfort and use the structure according to its purpose of use.

Section 5. Competence of the Cabinet in the Field of Construction

(1) For enforcement of the Law the Cabinet shall:

1) issue general construction regulations determining:

a) the classification and division of structures into groups depending on the level of complexity of construction and the potential impact on human life, health and the environment,

b) general procedures of the construction process depending on the relevant group of the structure,

c) cases when performance of engineering research work is necessary,

d) cases when expert-examination of a building design is necessary, and the procedures for performance thereof,

e) cases when author supervision and building supervision is necessary, as well as the procedures for author supervision and building supervision and the procedures for the drawing up and the content of a building supervision plan,

f) the procedures and conditions for building control, as well as the rights and duties of building inspectors,

g) the principles and documents, on the basis of which a decision should be taken regarding putting in order or demolishing of such structure which has completely or partially collapsed, is dangerous or spoils the landscape;

2) issue special construction regulations determining:

a) the construction process procedures, the institutions involved in the construction process and the responsible building specialists,

b) the documents necessary for the construction process and their content,

c) the conditions to be included in the construction permit,

d) the procedures for co-ordinating deviations,

e) the scope of expert-examination of a building design,

f) the institution responsible for the control and rule of law of the construction process,

g) the procedures for the preservation of a structure,

h) the procedures for conducting a survey of the layout of a structure and the procedures by which a structure shall be accepted for service,

i) the time periods of guarantees for construction works after a structure has been accepted for service;

3) issue construction standards in which technical requirements in relation to structures and their elements and the requirements for environmental accessibility in relation to structures shall be determined;

4) issue regulations regarding competence evaluation of building specialists and supervision of independent practice, determining:

a) the conditions by which a certificate for an independent practice in the field of architecture and construction shall be issued to, registered and cancelled for natural persons, as well as the procedures for the issuance, registration and cancellation of a certificate, suspending and restoring the operation thereof,

b) the necessary length of practical work experience in the field of architecture and construction in order to apply for a certificate for an independent practice in the field of architecture and construction,

c) the conditions for organising a competence examination and the content of a competence examination,

d) the procedures for supervising an independent practice,

e) the amount of paid services, the payment procedures and the procedures for the use of the received resources,

f) the authorisation of the authority examining the competence,

g) the extent of information to be included in the register of building specialists and the procedures for examination thereof;

5) issue regulations determining:

a) the requirements to be set for building inspectors, the procedures for registration of building inspectors and the procedures by which building inspectors shall be excluded from the register,

b) the content of the data of the register of building inspectors,

c) the procedures for supervising the professional activity of building inspectors;

6) determine the procedures for the creation and maintenance of a construction information system, the procedures for issuing documents, the content of the construction information system and the procedures for the circulation of information included therein;

7) determine the procedures and conditions for registration and re-registration of merchants in the register of construction merchants, exclusion from the register, as well as the content and the procedures for the use of data of the register, the amount and payment procedures of the State fee to be paid for registration activities;

8) determine the procedures for the establishment and operation of the Construction Council of Latvia, as well as the functions of the secretariat of the Council;

9) determine the procedures for insuring professional civil legal liability of building specialists and for insuring civil legal liability of building contractors and the minimum limit of liability;

10) determine the procedures by which market supervision of construction products shall be performed, samples of construction products shall be requested and received within the framework thereof, laboratory expert-examinations or expert-examinations of another kins shall be performed and the cases when expenses for performing respective expert-examinations shall be covered by the manufacturer or distributor of construction materials;

11) determine the procedures by which the European technical assessment institution shall be selected, as well as the procedures by which a European technical assessment shall be issued;

12) issue regulations regarding classification of construction merchants, determining:

a) the information to be submitted for receipt of a classification document,

b) the types of and criteria for classification of construction merchants,

c) the conditions for the receipt of a single and temporary classification document,

d) the conditions for amending or revocation of a decision to grant a classification document,

e) the authorisation of the classification authority,

f) information regarding the procedures for placing, maintenance, updating and checking of classifications of construction merchants.

(2) The regulations referred to in Paragraph one, Clause 2 of this Section shall be issued in relation to:

1) buildings;

2) motorways and streets (State and local government);

3) railway structures (State, local government and private);

4) electronic communications structures;

5) energy production, storage, transmission and distribution structures;

6) hydrotechnical and land amelioration structures;

7) port hydrotechnical structures;

8) structures related to radiation safety;

9) structures in the territorial waters and exclusive economic zone of the Republic of Latvia;

10) others, particularly non-classified structures.

Section 6. Competence of State Administrative Institutions in the Field of Construction

(1) The ministry responsible for the construction sector shall perform general monitoring and co-ordination of construction, draw up a single State policy in the field of construction and ensure the implementation of such policy. In order to perform such tasks, the ministry shall:

1) draw up policy planning documents in the field of construction;

2) draw up proposals for improvement of the system of the laws and regulations governing construction;

3) draw up the regulations referred to in Section 5 of this Law;

4) ensure operation of the construction information system necessary for the monitoring and control of construction.

(2) The competence of the ministry responsible for the construction sector may also include other duties in the field of construction provided for in laws and regulations.

(3) Sectoral ministries, according to the competence laid down in their by-laws in relation to constructing the structures referred to in Section 5, Paragraph two, Clauses 2, 3, 4, 5, 6, 7, 8 and 9 of this Law, shall submit the following information to be included in the special construction regulations to the ministry responsible for the construction sector:

1) the construction process procedures and the institutions involved in the construction process;

2) the documents necessary for the construction process and their content;

3) the conditions to be included in the construction permit;

4) the institution responsible for the supervision and rule of law of the construction process.

(4) Sectoral ministries or the authorities subordinate thereto, according to the competence laid down in their by-laws in the cases provided for in laws and regulations, shall take decisions on construction intention in carrying out the functions of the building authority laid down in Section 12, Paragraph three, Clauses 1, 3, 4, 5, 6, 7, 9 and 10 of this Law, as well as in applying the time periods for decision-making, settlement of disputes laid down in the law and the procedures for notification of a construction intention.

(5) An authority subordinate to the ministry responsible for the construction sector shall perform supervision and control of the market of construction products (manufacturing and trade locations, construction sites), checking the conformity of construction products with the performance attributes declared in the standards or technical provisions and the requirements laid down in laws and regulations, and shall take decisions binding to the participants of the market of construction products. The decisions taken by the authority may be appealed in accordance with the procedures laid down in the Administrative Procedure Law.

(6) In performing supervision and control of the market of construction products, the authority has the right:

1) to request and receive free of charge information and documents (including technical documentation, declaration of the performance attributes, type-examination certificate, testing reports);

2) to request and receive free of charge sample construction products, to perform sample checks on site (pre-testing), also opening the packaging for such purpose and performing sample testing, as well as to organise laboratory expert-examination or expert-examination of another kind in order to determine the conformity of the construction product with the requirements laid down;

3) to prohibit or suspend placing on the market, offering or selling of construction products for a time period which is necessary in order to assess the conformity of the construction products with the requirements laid down or the performance attributes declared, in order to perform checks and expert-examinations, as well as until the final decision is taken;

4) after checks are performed to provide recommendations to market participants regarding the necessary improvements.

Section 7. Competence of the Local Government

For enforcement of the Law the local government shall:

1) ensure the rule of law of the construction process:

a) by establishing a building authority in its territory or by reaching an agreement with other local governments regarding establishment of a joint building authority and by ensuring the resources necessary for the operation of the building authority,

b) by taking decisions on contested administrative acts, actual action and decisions of the building authority of the local government, to be contested in accordance with laws and regulations;

2) take decisions on further action involving the structures in its territory, which have completely or partially collapsed or are in such condition that their use is dangerous or they spoil the landscape;

3) in cities – employ an architect who monitors the conformity with the principle of architectonic quality;

4) ensure access to information in the construction information system;

5) in accordance with the procedures laid down in this Law inform the public regarding the construction intentions received.

Section 8. Construction Council of Latvia

(1) The Construction Council of Latvia shall:

1) assess draft construction policy planning documents and draft laws and regulations in the field of construction and provide an opinion on them to the ministry responsible for the construction sector, as well as participate in improving the laws and regulations governing construction;

2) draw up proposals for the development of the construction sector, including for determination of priority fields, improvement of the vocational education system of building specialists and improvement of their competence.

(2) The ministry responsible for the construction sector shall carry out the functions of the secretariat of the Construction Council of Latvia.

Section 9. Essential Requirements to be Set for a Structure

(1) A structure shall be designed and built according to the type of use thereof.

(2) During the whole economically justified term of service the structure and its elements must conform to the following essential requirements:

1) mechanical strength and stability;

2) fire safety;

3) hygiene, harmlessness and environmental protection;

4) safety of use and accessibility of the environment;

5) acoustics (noise protection);

6) energy efficiency;

7) sustainable use of natural resources.

(3) Deviations from technical requirements of the laws and regulations governing construction in a building design after its co-ordination with the relevant State and local government authorities shall be permissible in the following cases:

1) conformity with the requirements of Paragraph two of this Section is ensured by alternative technical solutions;

2) in renewing, rebuilding or restoring buildings with cultural and historical value, conformity with the relevant requirements is technically impossible or the building would lose its cultural and historical value due to conformity therewith.

Section 10. Construction Products

(1) It shall be permitted to offer construction products on the market of Latvia, as well as to build them permanently in structures, if they are valid for the intended use, ensure the fulfilment of the essential requirements set for the structure and conform to the requirements of the laws and regulations governing construction.

(2) If it is detected that a construction product does not conform to the requirements laid down or the performance attributes declared, the authority performing supervision and control of the market of construction products may take the following decisions:

1) to assign the market participant to eliminate the detected non-conformities;

2) to assign the market participant to take measures for ensuring the conformity of the construction product;

3) to prohibit to integrate the construction product in the structure;

4) to assign the market participant to withdraw the construction product from trade, to revoke from consumers and users;

5) to prohibit or suspend placing on the market or selling of construction products.

(4) Appealing of the decisions referred to in Paragraph two of this Section shall not suspend their operation.

Section 11. Division of Structures

Structures shall be divided into buildings and engineering structures. The criteria for a more detailed grouping of structures and the conditions for submitting documents, as well as for the performance of construction work within the scope of each group of structures shall be determined by general construction regulations.

Section 12. Building Authority

(1) A building authority is a local government authority or unit or an authority established by several local governments.

(2) A building authority shall employ specialists of whom at least one is an architect and one – a building inspector.

(3) A building authority, according to the competence thereof, shall:

1) control the construction process and its conformity with the requirements of this Law and other laws and regulations governing construction;

2) provide information regarding the conditions for the use and building of a territory, as well as the engineering networks existing in the territory;

3) inform regarding legal background for the construction occurring and provide information regarding the structure;

4) examine submissions and take decisions on construction intention;

5) examine the plan of construction supervision;

6) examine alternative technical solutions for ensuring the accessibility of the environment, if necessary, requesting an opinion of experts of non-governmental organisations of the relevant field if conformity with the technical requirements of the construction standards is not possible in the building design;

7) accept structures for service;

8) examine submissions and take decisions to change the type of use of a structure or its part without rebuilding;

9) provide consultations regarding the construction process procedures;

10) perform other activities related to the construction process and its conformity with the requirements of laws and regulations;

11) register the construction permits issued by other institutions;

12) provide consultations regarding construction possibilities in the relevant territory.

(4) Depending on the construction intention the building authority shall take a decision within the following time periods:

1) within one month – on the issue referred to in Section 14, Paragraph three, Clause 1 of this Law;

2) within 14 days – on the issue referred to in Section 14, Paragraph three, Clause 2 of this Law;

3) within seven days – on the issue referred to in Section 14, Paragraph three, Clause 3 of this Law.

(5) The building authority shall make a note in the construction permit regarding execution of the conditions included therein within seven days from the day when documents certifying execution of all the referred-to conditions were submitted to the building authority.

Section 13. Building Specialists

(1) Building specialists shall be persons who have acquired the right to an independent practice in the field of architecture or construction in regulated professions.

(2) The following persons may acquire the right to an independent practice in the field of architecture or construction:

1) who have acquired the second level vocational higher education in a study programme in architecture;

2) who have acquired the knowledge and skills necessary for an independent practice;

3) who have received a certificate of architect’s practice in accordance with the procedures laid down in laws and regulations.

(3) The right to an independent practice in the field of construction in the profession of a construction engineer may be acquired in the following specialties:

1) engineering research;

2) design;

3) construction works management;

4) construction supervision;

5) construction expert-examination.

(4) The right to an independent practice in the profession of a construction engineer in the specialties referred to in Paragraph three, Clauses 3 and 4 of this Section may be acquired by a person who:

1) has acquired the first level vocational higher education in a study programme of a construction engineer;

2) has acquired the knowledge and skills necessary for an independent practice;

3) has received a certificate of building practice in accordance with the procedures laid down in laws and regulations.

(5) The right to an independent practice in the profession of a construction engineer in the specialties referred to in Paragraph three, Clauses 1, 2, 3, 4 and 5 of this Section may be acquired by a person who:

1) has acquired the second level vocational higher education in a study programme of a construction engineer;

2) has acquired the knowledge and skills necessary for an independent practice;

3) has received a certificate of building practice in accordance with the procedures laid down in laws and regulations.

(6) Building specialists may perform a construction expert-examination only in such fields in which they have the right to an independent practice.

(7) Building specialists shall regularly improve their professional qualification.

(8) Information regarding the certificate of an architect’s practice and a certificate of construction practice shall be included in the register of building specialists in accordance with the procedures laid down in laws and regulations

(9) A certificate necessary in the specialities laid down in this Law for persons for the acquisition of the right to an independent practice in the field of architecture in the profession of an architect or in the field of construction in the profession of a construction engineer shall be granted without time limitation. In the cases provided for in laws and regulations assessment of the competence of a building specialists shall be repeated.

(10) Building specialists have a duty to insure their professional liability for the damage caused in the construction process as a result of their actions or failure to act to the life, health or property of other participants of construction and third parties, as well as to the environment.

(11) Building specialists shall perform the activities laid down in this Law in the process of designing and construction on the basis of a written contract.

(12) The requirements of Paragraph four of this Section shall not apply to master craftsmen, if they perform or manage building trade works in accordance with the Law On Trade Activities. In the cases provided for in special construction regulations construction works in individual structures may be managed also by other persons not referred to in this Section.

(13) The assessment of the competence and the supervision of an independent practice of a building specialist shall be delegated and payment for the performance of the referred-to tasks shall be delegated in accordance with the procedures laid down in the State Administrative Structure Law.

Section 14. Construction Intention

(1) A person shall propose construction by submitting an application of a construction intention, as well as the documents provided for in general construction regulations and special construction regulations to the building authority.

(2) An institution or official determined by a local government shall notify the public regarding a construction intention within three working days from the day when the application of the relevant construction intention was received, publishing the relevant information on the website of the local government and in the construction information system. The following shall be indicated in the publication:

1) the location where the construction intention will be implemented;

2) the type of the intended structure.

(3) Upon examining an application of a construction intention, the building authority depending on the type of the construction intention shall take a decision:

1) to issue a construction permit or to refuse to issue a construction permit;

2) to accept the construction intention, making a note in the certification card, or to refuse to accept the construction intention according to a simplified procedure;

3) to accept the construction intention, making a note in the explanatory memorandum of the construction intention, or to refuse to accept the construction intention.

(4) In the cases provided for in the special construction regulations (installation of engineering network circuits, installation of engineering networks in a protection zone or within the limits of red lines) a person shall commence installation of engineering networks after he or she has notified the building authority regarding the construction intention and has received other permits, if such are necessary in accordance with laws and regulations.

(5) An institution or official determined by a local government shall notify the public regarding a decision, which has been taken upon examining an application of a construction intention in the case referred to in Paragraph three, Clause 1 of this Section, publishing a notification on the website of the local government and in the construction information system within three working days from the day when the decision was taken. The following shall be indicated in the publication:

1) the location where the construction intention will be implemented;

2) the type of the intended structure;

3) the day when the construction permit or a refusal to issue a construction permit enters into effect.

(6) The commissioning party has a duty to inform the public regarding the received construction permit in the cases laid down in laws and regulations within five days by placing a construction board on the plot of land on which construction is permitted. The content of information shall be determined by the Cabinet. Such measure shall be informative in nature.

(7) The commissioning party may inform such owners (possessors) of immovable properties individually in writing regarding the received construction permit whose immovable properties are adjacent to the plot of land on which construction is permitted. Such measure shall be informative in nature.

(8) A construction permit or a refusal to issue a construction permit may be contested or appealed in accordance with the procedures laid down in the Administrative Procedure Law within a month from the day referred to in Paragraph five, Clause 3 of this Section.

Section 15. Construction Permit

(1) A construction permit shall be issued if:

1) the construction intention conforms to the provisions for the use and building of a territory included in the spatial plan, local plan and detailed plan of a local government, except cases when a construction intention is related to an object of national interest;

2) the construction intention has been co-ordinated with the owner or legal possessor of the plot of land and the owner or legal possessor of the structure in case if construction is intended in an existing structure, or the owner of the plot of land has been informed in case when it is requested by laws and regulations;

3) a building design in a minimum composition has been drawn up in accordance with the requirements of the laws and regulations governing construction, except cases when drawing up of a relevant design is not necessary.

(2) A construction permit shall not be issued if:

1) the construction intention does not conform to any of the conditions of Paragraph one of this Section or other requirements of laws and regulations;

2) an initial assessment of the intended construction or an environmental impact assessment in case when it is determined by laws and regulations has not been performed.

(3) A decision on a construction permit shall enter into effect from the moment it has been notified to the addressee. If information regarding a decision on a construction permit has been published in conformity with the conditions of Section 14, Paragraph five of this Law, such decision in relation to other persons shall be deemed notified from the moment it entered into effect.

(4) Additional conditions in relation to the level of detail of a building design may be determined in the regulations of the local government for building of the territory, if they are necessary for a structure to blend with the landscape or the urban environment.

(5) In addition to the cases laid down in the Administrative Procedure Law a construction permit may be revoked in accordance with the procedures laid down in laws and regulations by the issuer thereof, if the commissioning party who is the owner, possessor or user of the relevant immovable property upon whose assignment construction is carried out on the basis of the contract entered into, does not conform to that laid down in this Law or other laws and regulations governing construction.

(6) Contesting or appeal of a construction permit issued for construction of an object of national interest shall not suspend the operation thereof.

Section 16. Designing

(1) After receipt of a construction permit the fulfilment of the conditions of the construction permit is commenced, ensuring drawing up of a building design in the extent laid down in general and special construction regulations, as well as in conformity with the provisions for the use and building of a territory included in the spatial plan, local plan and detailed plan of a local government (if it is necessary in accordance with laws and regulations). Designing at the risk of the commissioning party may be continued also during the time period when the construction permit has been contested or appealed.

(2) The person drawing up the building design shall be responsible for the conformity of the building design or its documentation with the requirements of the laws and regulations governing construction, of construction norms, as well as contracts entered into by the designer and the commissioning party according to civil legal procedures.

(3) The person drawing up the building design has the right to perform author supervision. Parties shall reach an agreement regarding the conditions thereof by entering into a written contract. General construction regulations shall provide for cases when author supervision is mandatory.

Section 17. Construction Works

(1) Construction works may be commenced after the building authority has made a note in the construction permit regarding the fulfilment of all the conditions included therein, the fulfilment of the conditions for commencing construction works and the construction permit has ceased to be subject to appeal. In the case referred to in Section 15, Paragraph six of this Law construction works may be commenced after the building authority has made a note in the construction permit regarding the fulfilment of the design conditions included therein and the fulfilment of the conditions for commencing construction works.

(2) Construction works shall be organised and carried out according to a building design and conditions of a construction permit, as well as in conformity with the restrictions and requirements laid down in laws and regulations in order to prevent causing harm to the environment or to make it as small as possible and the consumption of resources would be economically and socially justified.

(3) If construction works are stopped or discontinued and, therefore, the structure becomes dangerous to human life, health or the environment or if dangerous damages may occur in constructions of the structure, the person who proposed the construction shall perform conservation in accordance with the procedures laid down in laws and regulations.

(4) Construction works for the carrying out of which a certification card or explanatory memorandum is necessary may be commenced after the building authority has made a note in the referred-to documents regarding harmonisation.

Section 18. Construction Control

(1) Construction control shall be performed by the building authority and building inspectors – persons employed at the building authority who are registered with the register of building inspectors.

(2) Construction works which have been commenced or are carried out without a construction permit or before a note has been made in the construction permit, certification card or explanatory memorandum regarding fulfilment of the relevant conditions, in cases when the relevant decisions are necessary in accordance with laws and regulations, as well as construction works which do not conform to the building design and the requirements of laws and regulations shall be qualified as unauthorised construction. Unauthorised construction is also activity or construction works, which have been commenced without a relevant design documentation, if such is necessary in accordance with laws and regulations.

(3) A building inspector has the right to see and inspect a structure during construction works, as well as during service thereof.

(4) In performing construction control, building inspectors shall inspect whether:

1) construction works have been commenced in accordance with the requirements of laws and regulations;

2) construction works and construction products conform to the building design and the requirements of laws and regulations;

3) a building or its part is used according to the designed type of use.

(5) If a building inspector in accordance with Paragraph four, Clause 1 of this Section detects unauthorised construction, the building inspector shall stop the construction works and write a relevant opinion, and the building authority shall take one of the following decisions:

1) to renew the previous condition, if construction of the particular object in the relevant territory is precluded by laws and regulations;

2) on permission to perform construction after fulfilment of the requirements of the laws and regulations governing construction, but in cases when harm to the environment has been caused as a result of unauthorised construction – also after elimination thereof in accordance with the procedures laid down in the laws and regulations governing environmental and nature protection.

(6) If in accordance with Paragraph four, Clause 2 of this Section a building inspector detects deviations from the building design submitted to the building authority, the building inspector shall stop the construction works and write a relevant opinion. The building authority may take a decision on permission to continue the construction works and a possibility to change the conditions of the construction permit, if changes in the building design are co-ordinated with the designer and do not affect the dimension, layout or type of use of the structure.

(7) If a building inspector has justified doubts regarding conformity of construction products with the requirements of laws and regulations, the building inspector shall inform the authority which carries out the supervision and control of the market of construction products.

(8) The building authority may take a decision to revoke a construction permit and on a duty to submit a new application of construction intention, if changes concern the dimension, layout or type of use of the structure and it is permitted by the regulations of a local government for the use and building of the territory.

(9) The building authority shall take a decision to tear down parts of the building or dismantle constructions not conforming to the regulations of a local government for the use and building of the territory.

(10) If a building inspector in accordance with Paragraph four, Clause 3 of this Section detects that a structure or its part is used contrary to the designed type of use, the building inspector shall write a relevant opinion to be submitted to the local government for it to take a decision on administrative punishment.

(11) Contesting or appealing of a decision to suspend construction, to discontinue construction works or to revoke a construction permit or to recognise as invalid shall not suspend the operation of such decision.

Section 19. Responsibility in Construction

(1) Participants of a construction process (landowner, owner of the structure, person who drew up the building design, performer of construction works, building supervisor and construction expert) have a duty to conform to the requirements of laws and regulations in the building process.

(2) A landowner shall be responsible for the commencement or performance of construction work without a construction permit before a note has been made in the construction permit, certification card or explanatory memorandum regarding fulfilment of the relevant conditions, if the relevant decisions are necessary for commencement of construction. The landowner shall also be responsible for choosing a person who draws up the building design, performer of construction work and building supervisor conforming to laws and regulations.

(3) If there is a structure belonging to another person on the plot of land or a structure belong to another person is being built on the plot of land, the owner of the structure shall be responsible for the commencement or performance of construction work without a construction permit before a note has been made in the construction permit, certification card or explanatory memorandum regarding fulfilment of the relevant conditions, if the relevant decisions are necessary for commencement of construction. The owner of the structure shall also be responsible for choosing a person who draws up the building design, performer of construction work and building supervisor conforming to laws and regulations.

(4) The person who draws up the building design shall be responsible for the conformity of the extent and content of the building design with the requirements of the commissioning party, this Law and other laws and regulations, as well for author supervision.

(5) The performer of construction work shall be responsible for the conformity of the requirements of laws and regulations at the construction site and for the conformity of the structure or its part created as a result of construction work with the building design and the requirements of the commissioning party, this Law and other laws and regulations, as well as for choosing construction products conforming to the requirements of laws and regulations and integration technologies thereof.

(6) A building supervisor shall be responsible for supervision of construction work, for conformity of the structure or its part, during construction of which the building supervisor carried out his or her duties, with the building design and the requirements of the commissioning party, this Law and other laws and regulations, as well as timely performance of the measures included in the plan of building supervision.

(7) A building supervisor shall be responsible for the content of an expert-examination opinion and the validity of the conclusions included therein.

(8) A person participating in construction process has a duty to compensate the losses caused by his or her action or failure to act to other participants of the construction process and third parties in accordance with the procedures laid down in this Law and The Civil Law.

(9) If a construction permit is necessary for performance of construction work, written employment contracts or contracts for work performance shall be entered into between participants of construction, except family members and related of the first and second degree, regarding execution of work or provision of services. If a construction permit is necessary for the performance of construction work, the duties and rights of participants of construction in relation to the form of entering into contracts shall be determined by The Civil Law, Labour Law and other laws and regulations.

(10) If in accordance with Paragraph nine of this Section entering into a written contract is necessary, however, participants of construction – natural persons – have not entered into a written contract, then upon deciding whether to hold a natural person liable according to the law or take another decision unfavourable thereto, in addition to the circumstances laid down in other laws and regulations, if the relevant person has indicated and justified them, also the circumstance whether work has been carried out and service has been provided without remuneration within the scope of personal relationship shall be evaluated.

Section 20. Insurance in Construction

(1) A performer of construction work has a duty to insure its civil legal liability for harm caused to the life or health of a third party or losses caused to the property as a result of its activity or failure to act. If a construction permit is necessary for the performance of construction works, the civil legal liability of the performer of construction work shall be insured for the whole term of performance of construction work.

(2) A contract regarding insuring civil legal liability shall be entered into either in relation to all construction objects and renewed each year, or anew, or also in relation to each individual construction object.

(3) Losses caused to the property of the third party are evaluated in conformity with the laws and regulations regarding insurance. The amount of the insurance compensation shall be determined by agreement of the parties.

(4) The issuer of the construction permit shall verify existence of the mandatory insurance of the civil legal liability of the performer of construction works.

Section 21. Service of a Structure

(1) A structure and its elements during its service shall be maintained in a way to ensure the conformity with the principles of construction referred to in Section 4 of this Law and the fulfilment of the essential requirements laid down in Section 9.

(2) After completion of construction work the person who proposed construction shall ensure that surveying of the layout of the structure is performed and the structure is accepted for service.

(3) A structure to be rebuilt or renewed, as well as engineering networks and transport engineering structures to be newly built may be used prior to accepting them for service, if the requirements of laws and regulations are conformed to, including the necessary measures for protecting human life, health and the environment have been taken and the issue regarding the use of the structure prior to accepting it for service has been co-ordinated with the building authority.

(4) After acceptance of a structure for service the structure may be used only for the designed type of the use.

(5) If a structure has fully or partially collapsed or is in such technical condition that it is dangerous or spoils the landscape, its owner in accordance with a decision of the local government must put it in order or demolish it in accordance with Section 1084 of The Civil Law.

(6) If the owner of a structure has not carried out the decision of the local government, execution thereof in relation to the part regarding prevention of the danger caused by the structure may be ensured by the local government. Appeal of the decision shall not suspend the operation thereof in relation to the part regarding prevention of the danger. The costs related to the putting in order or demolition of the structure shall be covered by the owner of the structure.

Section 22. Register of Construction Merchants

(1) In order to conduct commercial activity in one or several fields of construction, a merchant shall register with the register of construction merchants.

(2) A merchant is entitled to operate in such fields of construction in which it has the relevant building specialists.

(3) Information regarding the merchant, its building specialists, data characterising the merchant, information regarding the violations detected in construction and violations of the provisions of the register shall be included in the register of construction merchants.

(4) The register of construction merchants shall be accessible to the public, except data access to which is restricted in accordance with the laws and regulations regarding commercial secret and data protection.

Section 23. Classification of Construction Merchants

(1) In order to apply for the performance of such construction work which are fully or partially financed from the resources of a legal person governed by public law, resources of policy instrument of the European Union or of other foreign financial assistance, if the commissioning party is a legal person governed by public law or its institution, a construction merchant must receive a classification document.

(2) Construction merchants shall be classified, assessing their financial and economic indicators, technical criteria, as well as professional experience.

(3) A construction merchant is entitled to apply to a single or temporary classification document.

(4) Information regarding classification of construction merchants shall be included in the construction information system and shall be accessible to the public.

Section 24. Construction Information System

(1) The construction information system is a State system, which includes the information and documents necessary for the construction process and its control and ensures circulation of information among public administration, control institutions and participants of construction, as well as involvement of the public in the construction process. The construction information system shall be created and maintained by the ministry responsible for the construction sector.

(2) The construction information system shall include:

1) information necessary for commencing construction;

2) information regarding the intended construction, decisions taken in relation thereto and entering into force thereof;

3) information regarding conditions of a construction permit and a list of the document certifying fulfilment thereof;

4) information regarding supervision of the construction process;

5) information necessary for accepting a structure for service;

6) information regarding participants of construction and violations of their professional activity;

7) the registers provided for in this Law and other laws regarding construction, energy performance of dwellings and buildings;

8) other information necessary for enforcement of this Law.

(3) The construction information system shall accessible to the public free of charge, except data access to which is restricted in accordance with the laws and regulations regarding commercial secret and data protection.

Transitional Provisions

1. With the coming into force of this Law the Construction Law (Latvijas Republikas Saeimas un Ministru Kabineta Ziņotājs, 1995, No. 20; 1997, No. 7, 22; 2002, No. 7; 2003, No. 6, 8; 2004, No. 9; 2005, No. 8; 2006, No. 7, 13; 2008, No. 3; 2009, No. 14; Latvijas Vēstnesis, 2010, No. 205; 2012, No. 195; 2013, No. 87, 124) is repealed.

2. Until the day when the Cabinet regulations substituting the relevant Cabinet regulations come into force, but not longer than until 1 July 2014, the following Latvian construction standards shall be applicable insofar as they are not in contradiction with this Law:

1) LBN 002-01 “Thermotechnics of Building Envelopes”;

2) LBN 003-01 “Construction Climatology”;

3) LBN 005-99 “Rules on Engineering Research in Construction”;

4) LBN 006-00 “Essential Requirements for Structures”;

5) LBN 016-11 “Construction Acoustics”;

6) LBN 201-10 “Fire Safety of Structures”;

7) LBN 202-01 “Content and Drawing up of Construction Design”;

8) LBN 203-97 “Design Standards of Concrete and Reinforced Concrete Structures”;

9) LBN 205-97 “Design Standards of Masonry and Reinforced Masonry Structures”;

10) LBN 206-99 “Design Standards of Timber Structures”;

11) LBN 207-01 “Geotechnics. Foundations and Bases of Structures”;

12) LBN 208-00 “Public Buildings and Structures”;

13) LBN 209-09 “Low-rise Residential Houses”;

14) LBN 211-08 “High-rise Multi-apartment Residential Buildings”;

15) LBN 214-01 “Geotechnics. Foundations and Bases of Piles”;

16) LBN 221-98 “Internal Water-main and Sewage of Buildings”;

17) LBN 222-99 “External Networks and Structures of Water Supply”;

18) LBN 223-99 “External Networks and Structures of Sewage”;

19) LBN 224-05 “Land Amelioration Systems and Hydrotechnical Structures”;

20) LBN 229-06 “Hydrotechnical Structures of Hydroelectric Power Plants”;

21) LBN 231-03 “Heating and Ventilation of Residential and Public Buildings”;

22) LBN 241-03 “Internal Systems of Gas-mains and Gas Installations”;

23) LBN 242-02 “External Networks of Gas Distribution and Users”;

24) LBN 261-07 “Building of Internal Electrical Installations of Buildings”;

25) LBN 262-05 “Electronic Communisations Networks”;

26) LBN 501-06 “Procedures for Determination of Construction Costs”;

27) LBN 305-01 “Geodetic Works in Construction”;

28) LBN 310-05 “Work Performance Design”;

29) LBN 405-01 “Technical Survey of Structures”.

3. Persons who until the day of coming into force of this Law have obtained the right to an independent practice in the field of construction in the profession of a building technician, however, have not acquired the education laid down in Section 13 of this Law are entitled to continue an independent practice in engineering research, design, managing of construction works, building supervision or building expert-examination not later than until 31 December 2017. If on the day of coming into force of such legal norm five years or less have remained until the person reaches the retirement age laid down in law, he or she shall be allowed to continue an independent practice in the fields referred to in this Paragraph until reaching the retirement age, without conforming to the requirements for education laid down for building specialists in Section 13 of this Law. If a person wishes to continue an independent practice in engineering research, design, managing of construction works, building supervision or building expert-examination after reaching the retirement age, he or she must conform to the requirements of Section 13 of this Law.

4. Persons who until the day of coming into force of this Law have obtained the right to an independent practice in the field of construction in the profession of a building engineer and who have acquired the first level vocational higher education in the study programme of a building engineer are entitled to continue an independent practice in engineering research, design or building expert-examination not later than until 31 December 2017. If on the day of coming into force of such legal norm five years or less have remained until the person reaches the retirement age laid down in law, he or she shall be allowed to continue an independent practice in the fields referred to in this Paragraph until reaching the retirement age, without conforming to the requirements for education laid down for building specialists in Section 13 of this Law. If a person wishes to continue an independent practice in engineering research, design or building expert-examination after reaching the retirement age, he or she must conform to the requirements of Section 13 of this Law.

5. Construction documentation which has been submitted to the responsible institution until the day of coming into force of this Law shall be applied the requirements of such laws and regulations which were in force on the day when the relevant documentation was submitted, insofar as more favourable requirements in relation to the content of the construction documentation and the time periods for decision-making have not been laid down in this Law.

6. The registers created in accordance with the Construction Law of 10 August 1995 and this Law from 1 May 2014 shall operate in accordance with the provisions of the laws and regulations governing them, shall be maintained and available within the scope of the construction information system.

7. Until making of amendments to laws and regulations the terms “rebuilding” and “change of the type of use” used in this Law shall conform to the term “reconstruction”, and the term “renewal” – to the term “renovation”.

8. Section 5, Paragraph three and Section 23 of this Law shall come into force from 1 January 2015. The pre-condition included in Section 23, Paragraph one of this Law for applying to the performance of construction works shall be applicable from the day when the relevant amendments to the Public Procurement Law and the Law On Procurements in the Field of Defence and Security, but not sooner than from 1 January 2015.

9. Building specialists who have received a certificate of an architect’s practice or building practice until the day of coming into force of this Law are entitled to continue an independent practice after expiry of the term of validity indicated in the certificate, if they conform to the requirements of this Law and provide the information to be included in the register of building specialists to the extent, within the time period and according to the procedures stipulated by the Cabinet.

This Law shall come into force on 1 February 2014.

This Law has been adopted by the Saeima on 9 July 2013.

President										A. Bērziņš

Rīga, 30 July 2013